Track disabling/enabling on MSRs for Elo touch monitors and all-in-one units

Magtek offers a USB Swipe and Insert Reader program that will allow you to send commands to program the MSR:

http://www.magtek.com/support/software/demo_programs/usb_swipe_insert.asp

There are two types of USB MSR’s – HID or Keyboard Emulation.  The technical reference manuals can be downloaded here:

USB HID Swipe Reader

http://www.magtek.com/docs/99875191.pdf

USB Keyboard Emulation Swipe Reader

http://www.magtek.com/docs/99875206.pdf

The mode of your MSR, KB or HID, will determine the commands that you need to send:

HID MSR Enable Track SET Commands:

Track 1 Data only           = 01 04 01

Track 2 Data only           = 01 04 04

Track 1 & 2 only            = 01 04 05

Track 1, 2, & 3               = 01 04 95

KB MSR Enable Track SET Commands:

Track 1 Data only           = 01 03 01

Track 2 Data only           = 01 03 04

Track 1 & 2 only            = 01 03 05

Track 1, 2, & 3               = 01 03 95

For example, in the Send Msg dialog box, to program the Keyboard Emulation MSR to read only Track 2 you would send:

01 03 04

After sending the above command you will then need to send:

02

The 02 command will cycle power to the MSR and save your changes.